ALI M3101 PC Upgrade Tool User Manual

--V3.94

PC upgrade 1->1

- At first you must check whether computer’s serial port had connected with STB by RS232

Run Upgrade.exe then you can get a window .

2. Click ‘Browse’ and select the software that you want to upgrade

3. In Upgrade Type you can select the upgrade type,normal it is all code.

- Upgrade Mode:Please select   eRom 1->1;

Plese make sure the Serial port is right.

Operate Mode:Normal it’s Upgrade.If you want to dump the software in STB to PC,you can sele Dump.

5. Now you can press Next and now it’s waiting for STB switch ON.

6.  Now you can switch on and you can see Synchronizing STBs…     Done.

If you cannot find this information.Please check your cable.

7. After a moment it will show download data.

8. Data download over,cleck Next to burn flash.

9. Now you can see the burning picture.Do NOT switch off  when Burning.

10.Now software had upgraded over.You can upgrade Next STB or Finish.

At this time,the STB will reboot.